Solution for HV-1

1)	The loop equation provides the nominal value of G..

G = F – ( A + B + C + D + E )

The maximum and minimum values are given by

where		 :	denotes a rightward (+) dimension-path segment

:	denotes a leftward (-) dimension-path segment

Max( X ) =	Xnominal + xupper

Min( X ) =	Xnominal - xlower

*

2)	Gmin must be > 0 to enable the brake to be assembled; a negative value would indicate spatial interference.

Gmax must be < D , where

D  = 	Range of actuator motion (see the drawing of the brake),

because the brake will be ineffective (part C won’t touch the part D) if the range of actuator motion does not exceed the maximum gap.

*

3)		G =	122.0 – (15.0 + 30.0 + 15.0 + 50.0 + 11.0)

1.0

Gmax =	122.1 – (14.8 + 29.7 + 14.8 + 49.9 + 10.9)

=	2.0

Gmin =	121.8 – (15.2 + 30.2 + 15.2 + 50.1 + 11.1)

=	0.0

Can the brake always be assembled? "Almost always" … because Gmin can be precisely zero only in a limiting sense, and it is formally unclear what would happen if the clearance were to be precisely zero. From a practical perspective, Gmin should be greater than zero by some sensible small amount (say 0.10 in this design), to insure assemble-ability and to insure also that the brake does not "drag" due to insufficient clearance.

Will the brake always work acceptably, assuming it can be assembled? Yes: Gmax is much smaller than the likely range of actuator motion.

*	*